GEOTRANS ENGINE

10xxxxxx.1

# LEGAL RESTRICTIONS:

This Reusable Software Component (RSC) contains data with Unlimited Government Rights.

# DESCRIPTION:

GEOTRANS ENGINE is a C language code component that performs general-purpose datum transformation and coordinate conversion operations.  Input coordinates that are defined with respect to a specified input datum and coordinate system, projection, or grid system are converted to a specified output datum and coordinate system, projection, or grid system.  Conversions between ellipsoid heights and geoid (or MSL) heights are also supported.

# CERTIFICATION LEVEL:

This RSC has been certified at level 4.  A level 4 component satisfies the criteria for reliability, testing, and documentation for the Army Reuse Center (ARC).  The component comes with test materials and a Reuse Manual that aids in integrating the component into a software system.

# LEVEL OF TESTING/ACCEPTANCE:

Unit and integration testing have been performed for the functions contained in this component.

# PURPOSE/INTENDED USE:

The purpose of GEOTRANS ENGINE is to provide a reusable component that supports the following coordinate conversions:

- Conversions between geodetic, geocentric, and local cartesian coordinate systems,

- Conversions between geodetic coordinates (latitude and longitude, in radians) and various types of map projection coordinates (easting and northing, in linear units),

- Conversions between geodetic coordinates and Military Grid Reference System (MGRS), United States National Grid (USNG), Global Area Reference System (GARS), or World Geographic Reference System (GEOREF) grid coordinates,

- Transformations of geodetic or geocentric coordinates between different global or local horizontal datums,

- Transformations between ellipsoid heights and geoid (or local MSL) heights, and

- Conversions that combine two or more of the above operations.

# HARDWARE/ENVIRONMENT CONSTRAINTS:

There are no hardware or environment constraints.  There are no limitations.

# FUNCTIONS:

Initialize_Engine – This function sets the initial state of the engine in preparation for coordinate conversion and/or datum transformation operations.

Get_Coordinate_System_Count – This function returns the number of different available coordinate systems (including projections and grid systems).

Get_Coordinate_System_Index – This function returns the index of the coordinate system with the specified code.

Get_Coordinate_System_Type – This function returns the type of the coordinate system with the specified index.

Get_Coordinate_System_Name – This function returns the name of the coordinate system with the specified index.

Get_Coordinate_System_Code – This function returns the code of the coordinate system with the specified index.

Set_Coordinate_System – This function sets the current input or output coordinate system to the specified type.

Get_Coordinate_System – This function returns the type of the current input or output coordinate system.

Get_Datum_Count – This function returns the number of different available datums.

Get_Datum_Index – This function returns the index of the datum with the specified code.

Get_Datum_Name – This function returns the name of the datum with the specified index.

Get_Datum_Code – This function returns the code of the datum with the specified index.

Get_Datum_Ellipsoid_Code – This function returns the code for the ellipsoid associated with the datum with the specified index.

Get_Datum_Type – This function returns the type of the datum with the specified index.

Get_Datum_Seven_Parameters – This function returns the seven parameters of the datum with the specified index.

Get_Datum_Three_Parameters – This function returns the three parameters of the datum with the specified index.

Get_Datum_Errors – This function returns the standard errors in X, Y, & Z of the datum with the specified index.

Get_Datum_Valid_Rectangle – This function returns the edges of the validity rectangle of the datum with the specified index.

Check_Datum_User_Defined – This function indicates whether the datum with the specified index is user defined.

Check_Valid_Datum – This function indicates whether the specified location is within the validity rectangle of the datum with the specified index.

Set_Datum – This function sets the current input or output datum to the datum with the specified index.

Get_Datum – This function returns the index of the current input or output datum.

Define_Datum – This function creates a new local 3-parameter or 7-parameter datum with the specified code, name, shift values (relative to WGS84), for 3-parameter datums standard error values and domain of validity, or for 7-parameter datums rotation values and a scale value.

Remove_Datum – This function deletes the user defined 3-parameter datum with the specified code.

Get_Ellipsoid_Count – This function returns the number of different available ellipsoids.

Get_Ellipsoid_Index – This function returns the index of the ellipsoid with the specified code.

Get_Ellipsoid_Name – This function returns the name of the ellipsoid with the specified index.

Get_Ellipsoid_Code – This function returns the code of the ellipsoid with the specified index.

Get_Ellipsoid_Parameters – This function returns the semi-major axis and flattening of the ellipsoid with the specified index.

Get_Ellipsoid_Eccentricity2 – This function returns the square of the eccentricity of the ellipsoid with the specified index.

Check_Ellipsoid_User_Defined – This function indicates whether the ellipsoid with the specified index is user defined.

Define_Ellipsoid – This function creates a new ellipsoid with the specified code, name, and semi-major axis and flattening values.

Remove_Ellipsoid – This function deletes the user defined ellipsoid with the specified code.

Set_Precision – This function sets the current output precision to the specified level.

Get_Precision – This function returns the current output precision level.

Set_Parameters – This function sets the parameters of the current input or output coordinate system to their default values.

Set_Geocentric_Coordinates – This function sets the current input or output Geocentric coordinates to the specified values.

Get_Geocentric_Coordinates – This function returns the current input or output Geocentric coordinates.

Set_Geodetic_Params – This function sets the current input or output Geodetic coordinate system parameters to the specified values.

Get_Geodetic_Params – This function returns the current input or output Geodetic coordinate system parameters.

Set_Geodetic_Coordinates – This function sets the current input or output Geodetic coordinates to the specified values.

Get_Geodetic_Coordinates – This function returns the current input or output Geodetic coordinates.

Set_GEOREF_Coordinates – This function sets the current input or output GEOREF coordinate string to the specified value.

Get_GEOREF_Coordinates – This function returns the current input or output GEOREF coordinate string.

Set_GARS_Coordinates – This function sets the current input or output GARS coordinate string to the specified value.

Get_GARS_Coordinates – This function returns the current input or output GARS coordinate string.

Set_Albers_Equal_Area_Conic_Params – This function sets the current input or output Albers Equal Area Conic projection parameters to the specified values.

Get_Albers_Equal_Area_Conic_Params – This function returns the current input or output Albers Equal Area Conic projection parameters.

Set_Albers_Equal_Area_Conic_Coordinates – This function sets the current input or output Albers Equal Area Conic projection coordinates to the specified values.

Get_Albers_Equal_Area_Conic_Coordinates – This function returns the current input or output Albers Equal Area Conic projection coordinates.

Set_Azimuthal_Equidistant_Params – This function sets the current input or output Azimuthal Equidistant projection parameters to the specified values.

Get_Azimuthal_Equidistant_Params – This function returns the current input or output Azimuthal Equidistant projection parameters.

Set_Azimuthal_Equidistant_Coordinates – This function sets the current input or output Azimuthal Equidistant projection coordinates to the specified values.

Get_Azimuthal_Equidistant_Coordinates – This function returns the current input or output Azimuthal Equidistant projection coordinates.

Set_Bonne_Params – This function sets the current input or output Bonne projection parameters to the specified values.

Get_Bonne_Params – This function returns the current input or output Bonne projection parameters.

Set_Bonne_Coordinates – This function sets the current input or output Bonne projection coordinates to the specified values.

Get_Bonne_Coordinates – This function returns the current input or output Bonne projection coordinates.

Set_BNG_Coordinates – This function sets the current input or output British National Grid coordinates to the specified values.

Get_BNG_Coordinates – This function returns the current input or output British National Grid coordinates.

Set_Cassini_Params – This function sets the current input or output Cassini projection parameters to the specified values.

Get_Cassini_Params – This function returns the current input or output Cassini projection parameters.

Set_Cassini_Coordinates – This function sets the current input or output Cassini projection coordinates to the specified values.

Get_Cassini_Coordinates – This function returns the current input or output Cassini projection coordinates.

Set_Cylindrical_Equal_Area_Params – This function sets the current input or output Cylindrical Equal Area projection parameters to the specified values.

Get_Cylindrical_Equal_Area_Params – This function returns the current input or output Cylindrical Equal Area projection parameters.

Set_Cylindrical_Equal_Area_Coordinates – This function sets the current input or output Cylindrical Equal Area projection coordinates to the specified values.

Get_Cylindrical_Equal_Area_Coordinates – This function returns the current input or output Cylindrical Equal Area projection coordinates.

Set_Eckert4_Params – This function sets the current input or output Eckert IV projection parameters to the specified values.

Get_Eckert4_Params – This function returns the current input or output Eckert IV projection parameters.

Set_Eckert4_Coordinates – This function sets the current input or output Eckert IV projection coordinates to the specified values.

Get_Eckert4_Coordinates – This function returns the current input or output Eckert IV projection coordinates.

Set_Eckert6_Params – This function sets the current input or output Eckert VI projection parameters to the specified values.

Get_Eckert6_Params – This function returns the current input or output Eckert VI projection parameters.

Set_Eckert6_Coordinates – This function sets the current input or output Eckert VI projection coordinates to the specified values.

Get_Eckert6_Coordinates – This function returns the current input or output Eckert VI projection coordinates.

Set_Equidistant_Cylindrical_Params – This function sets the current input or output Equidistant Cylindrical projection parameters to the specified values.

Get_Equidistant_Cylindrical_Params – This function returns the current input or output Equidistant Cylindrical projection parameters.

Set_Equidistant_Cylindrical_Coordinates – This function sets the current input or output Equidistant Cylindrical projection coordinates to the specified values.

Get_Equidistant_Cylindrical_Coordinates – This function returns the current input or output Equidistant Cylindrical projection coordinates.

Set_Gnomonic_Params – This function sets the current input or output Gnomonic projection parameters to the specified values.

Get_Gnomonic_Params – This function returns the current input or output Gnomonic projection parameters.

Set_Gnomonic_Coordinates – This function sets the current input or output Gnomonic projection coordinates to the specified values.

Get_Gnomonic_Coordinates – This function returns the current input or output Gnomonic projection coordinates.

Set_Lambert_Conformal_Conic_1_Params – This function sets the current input or output Lambert Conformal Conic (1 parallel) projection parameters to the specified values.

Get_Lambert_Conformal_Conic_1_Params – This function returns the current input or output Lambert Conformal Conic (1 parallel) projection parameters.

Set_Lambert_Conformal_Conic_1_Coordinates – This function sets the current input or output Lambert Conformal Conic (1 parallel) projection coordinates to the specified values.

Get_Lambert_Conformal_Conic_1_Coordinates – This function returns the current input or output Lambert Conformal Conic(1 parallel)  projection coordinates.

Set_Lambert_Conformal_Conic_2_Params – This function sets the current input or output Lambert Conformal Conic (2 parallel) projection parameters to the specified values.

Get_Lambert_Conformal_Conic_2_Params – This function returns the current input or output Lambert Conformal Conic (2 parallel)  projection parameters.

Set_Lambert_Conformal_Conic_2_Coordinates – This function sets the current input or output Lambert Conformal Conic (2 parallel) projection coordinates to the specified values.

Get_Lambert_Conformal_Conic_2_Coordinates – This function returns the current input or output Lambert Conformal Conic (2 parallel) projection coordinates.

Set_Local_Cartesian_Params – This function sets the current input or output Local Cartesian coordinate system parameters to the specified values.

Get_Local_Cartesian_Params – This function returns the current input or output Local Cartesian coordinate system parameters.

Set_Local_Cartesian_Conic_Coordinates – This function sets the current input or output Local Cartesian coordinates to the specified values.

Get_Local_Cartesian_Conic_Coordinates – This function returns the current input or output Local Cartesian coordinates.

Set_Mercator_Params – This function sets the current input or output Mercator projection parameters to the specified values.

Get_Mercator_Params – This function returns the current input or output Mercator projection parameters.

Set_Mercator_Coordinates – This function sets the current input or output Mercator projection coordinates to the specified values.

Get_Mercator_Coordinates – This function returns the current input or output Mercator projection coordinates.

Set_MGRS_Coordinates – This function sets the current input or output MGRS coordinate string.

Get_MGRS_Coordinates – This function returns the current input or output MGRS coordinate string.

Set_Miller_Cylindrical_Params – This function sets the current input or output Miller Cylindrical projection parameters to the specified values.

Get_Miller_Cylindrical_Params – This function returns the current input or output Miller Cylindrical projection parameters.

Set_Miller_Cylindrical_Coordinates – This function sets the current input or output Miller Cylindrical projection coordinates to the specified values.

Get_Miller_Cylindrical_Coordinates – This function returns the current input or output Miller Cylindrical projection coordinates.

Set_Mollweide_Params – This function sets the current input or output Mollweide projection parameters to the specified values.

Get_Mollweide_Params – This function returns the current input or output Mollweide projection parameters.

Set_Mollweide_Coordinates – This function sets the current input or output Mollweide projection coordinates to the specified values.

Get_Mollweide_Coordinates – This function returns the current input or output Mollweide projection coordinates.

Set_Neys_Params – This function sets the current input or output Ney’s (Modified Lambert Conformal Conic) projection parameters to the specified values.

Get_Neys_Params – This function returns the current input or output Ney’s (Modified Lambert Conformal Conic) projection parameters.

Set_Neys_Coordinates – This function sets the current input or output Ney’s (Modified Lambert Conformal Conic) projection coordinates to the specified values.

Get_Neys_Coordinates – This function returns the current input or output Ney’s (Modified Lambert Conformal Conic) projection coordinates.

Set_NZMG_Coordinates – This function sets the current input or output New Zealand Map Grid coordinates to the specified values.

Get_NZMG_Coordinates – This function returns the current input or output New Zealand Map Grid coordinates.

Set_Oblique_Mercator_Params – This function sets the current input or output Oblique Mercator projection parameters to the specified values.

Get_Oblique_Mercator_Params – This function returns the current input or output Oblique Mercator projection parameters.

Set_Oblique_Mercator_Coordinates – This function sets the current input or output Oblique Mercator projection coordinates to the specified values.

Get_Oblique_Mercator_Coordinates – This function returns the current input or output Oblique Mercator projection coordinates.

Set_Orthographic_Params – This function sets the current input or output Orthographic projection parameters to the specified values.

Get_Orthographic_Params – This function returns the current input or output Orthographic projection parameters.

Set_Orthographic_Coordinates – This function sets the current input or output Orthographic projection coordinates to the specified values.

Get_Orthographic_Coordinates – This function returns the current input or output Orthographic projection coordinates.

Set_Polar_Stereo_Params – This function sets the current input or output Polar Stereographic projection parameters to the specified values.

Get_Polar_Stereo_Params – This function returns the current input or output Polar Stereographic projection parameters.

Set_Polar_Stereo_Coordinates – This function sets the current input or output Polar Stereographic projection coordinates to the specified values.

Get_Polar_Stereo_Coordinates – This function returns the current input or output Polar Stereographic projection coordinates.

Set_Polyconic_Params – This function sets the current input or output Polyconic projection parameters to the specified values.

Get_Polyconic_Params – This function returns the current input or output Polyconic projection parameters.

Set_Polyconic_Coordinates – This function sets the current input or output Polyconic projection coordinates to the specified values.

Get_Polyconic_Coordinates – This function returns the current input or output Polyconic projection coordinates.

Set_Sinusoidal_Params – This function sets the current input or output Sinusoidal projection parameters to the specified values.

Get_Sinusoidal_Params – This function returns the current input or output Sinusoidal projection parameters.

Set_Sinusoidal_Coordinates – This function sets the current input or output Sinusoidal projection coordinates to the specified values.

Get_Sinusoidal_Coordinates – This function returns the current input or output Sinusoidal projection coordinates.

Set_Stereographic_Params – This function sets the current input or output Stereographic projection parameters to the specified values.

Get_Stereographic_Params – This function returns the current input or output Stereographic projection parameters.

Set_Stereographic_Coordinates – This function sets the current input or output Stereographic projection coordinates to the specified values.

Get_Stereographic_Coordinates – This function returns the current input or output Stereographic projection coordinates.

Set_Transverse_Cylindrical_Equal_Area_Params – This function sets the current input or output Transverse Cylindrical Equal Area projection parameters to the specified values.

Get_Transverse_Cylindrical_Equal_Area_Params – This function returns the current input or output Transverse Cylindrical Equal Area projection parameters.

Set_Transverse_Cylindrical_Equal_Area_Coordinates – This function sets the current input or output Transverse Cylindrical Equal Area projection coordinates to the specified values.

Get_Transverse_Cylindrical_Equal_Area_Coordinates – This function returns the current input or output Transverse Cylindrical Equal Area projection coordinates.

Set_Transverse_Mercator_Params – This function sets the current input or output Transverse Mercator projection parameters to the specified values.

Get_Transverse_Mercator_Params – This function returns the current input or output Transverse Mercator projection parameters.

Set_Transverse_Mercator_Coordinates – This function sets the current input or output Transverse Mercator projection coordinates to the specified values.

Get_Transverse_Mercator_Coordinates – This function returns the current input or output Transverse Mercator projection coordinates.

Set_UPS_Coordinates – This function sets the current input or output Universal Polar Stereographic (UPS) projection coordinates to the specified values.

Get_UPS_Coordinates – This function returns the current input or output Universal Polar Stereographic (UPS) projection coordinates.

Set_USNG_Coordinates – This function sets the current input or output USNG coordinate string.

Get_USNG_Coordinates – This function returns the current input or output USNG coordinate string.

Set_UTM_Params – This function sets the current input or output Universal Transverse Mercator (UTM) projection parameters to the specified values.

Get_UTM_Params – This function returns the current input or output Universal Transverse Mercator (UTM) projection parameters.

Set_UTM_Coordinates – This function sets the current input or output Universal Transverse Mercator (UTM) projection coordinates to the specified values.

Get_UTM_Coordinates – This function returns the current input or output Universal Transverse Mercator (UTM) projection coordinates.

Set_Van_der_Grinten_Params – This function sets the current input or output Van der Grinten projection parameters to the specified values.

Get_Van_der_Grinten_Params – This function returns the current input or output Van der Grinten projection parameters.

Set_Van_der_Grinten_Coordinates – This function sets the current input or output Van der Grinten projection coordinates to the specified values.

Get_Van_der_Grinten_Coordinates – This function returns the current input or output Van der Grinten projection coordinates.

Convert – This function converts the current input coordinates, according to the current input datum, coordinate system, and parameters, and the current output datum, coordinate system, and parameters.

Get_Conversion_Errors – This function returns the output 90% horizontal (circular), vertical (linear), and spherical error values for the most recent conversion.

Set_Conversion_Errors – This function sets the input 90% horizontal (circular), vertical (linear), and spherical error values to be used in subsequent conversions.

Get_Conversion_Status – This function returns the current input or output conversion status.

Get_Conversion_Status_String – This function returns a character string that corresponds to the current input or output conversion status.

Get_Return_Code_String – This function returns a character string that corresponds to the current input or output conversion status.

# EXAMPLE ApplicationS:

The following example illustrates how the Engine can be used to convert Geodetic coordinates to Mercator projection coordinates, both relative to the WGS 84 datum:

Note: Input coordinates and input/output parameters must be in radians.

## #include "engine.h"

## #define PI         3.14159265358979323e0

Parameters:

## long status;

## long datum_index;

## double ce90, le90, se90;

## Geodetic_Parameters input_params;

## Geodetic_Tuple input_coords;

## Mercator_Parameters output_params;

## Mercator_Tuple output_coords;

## Inputs:

## ce90 = 1.0;

## le90 = 1.0;

## se90 = 1.0;

## input_params.height_type = Ellipsoid_Height;

## input_coords.latitude = -35.0 * PI/180.0;

## input_coords.longitude = 75.0 * PI/180.0;

## input_coords.height = 0.0;

## output_params.central_meridian = 0.0;

## output_params.latitude_of_true_scale = 0.0;

## output_params.scale_factor = 1.0;

## output_params.false_easting = 0.0;

## output_params.false_northing = 0.0;

## Function Call:

## status = Initialize_Engine();

## status = Get_Datum_Index ("WGE", &datum_index);

## status = Set_Datum (Interactive, Input, datum_index);

## status = Set_Coordinate_System (Interactive, Input, Geodetic);

## status = Set_Geodetic_Params (Interactive, Input, input_params);

## status = Set_Geodetic_Coordinates (Interactive, Input, input_coords);

## status = Set_Conversion_Errors(Interactive, ce90, le90, se90);

## status = Set_Datum (Interactive, Output, datum_index);

## status = Set_Coordinate_System (Interactive, Output, Mercator);

## status = Set_Mercator_Params (Interactive, Output, output_params);

## status = Convert(Interactive);

## status = Get_Mercator_Coordinates (Interactive, Output, &output_coords);

## status = Get_Conversion_Errors (Interactive, &ce90, &le90, &se90);

## Outputs:

- output_coords.easting:	8348961.81

- output_coords.northing:	-4139372.76

- output_coords.height:		0.0

- ce90:		1.0

- le90		1.0

- se90		1.0